PI3EQX7841

PI3EQX7841 EVB Rev.A User Manual

By YT Tso and Anne Wu

- Introduce:

PI3EQX7841 is a low power high performance 5.0Gbps signal ReDriver designed for USB3.0 protocol. The device provides programmable Equalization (EQ) and De-Emphasis (DE) to optimize performance over a variety of physical mediums by reducing Inter-Symbol Interference (ISI)

Figure 1(a) top view of PI3EQX7841 EVB                                                 Figure 1(b) bottom view of PI3EQX7841 EVB

- Quick Start — For Pin Strap control:

To start-up the PI3EQX7841 EVB, complete the following steps:

- Short JP9 for power on EVB

- Check the head pin status setting follow Table 1

- Connect the JP6 on EVB to PC’s USB3.0 port through a USB3.0 Type A to B cable.

- Plug the USB3.0 device into EVB USB Type-A connector JP7 through USB3.0 cable

Table 1 Header pin is set as defaulted on EVB

| Header pin # | Pin name for PI3EQX7841 | Switch status | Remark |
| --- | --- | --- | --- |
| JP4 | EN# | Open | Chip Enable open Chip Disable short to Vdd |
| JP5 | I2C_EN | Short to GND | Pin strap control modeshort to GND I2C control modeshort to Vdd |
| JP8 | +5V | Short | Device Vbus 5V output |
| JP10 | EQ_A | Short to GND | Equalizer setting on Channel A = 3.3dB |
| JP11 | DE_A | Open | De-emphasis setting on Channel A = -3.5dB |
| JP12 | OS_A | Open | Output Swing setting on Channel A= 1000mVppd |
| JP13 | OS_B | Open | Output Swing setting on Channel B= 1000mVppd |
| JP14 | EQ_B | Short to GND | Equalizer setting on Channel B = 3.3dB |
| JP15 | DE_B | Open | De-emphasis setting on Channel B = -3.5dB |

Equalizer setting:

| Header pin # | Pin name for PI3EQX7841 | Switch status | Remark |
| --- | --- | --- | --- |
| JP10 | EQ_A | Short to GND |  |
| JP14 | EQ_B | Short to GND |  |

De-emphasis setting:

| Header pin # | Pin name for PI3EQX7841 | Switch status | Remark |
| --- | --- | --- | --- |
| JP11 | DE_A | Open |  |
| JP15 | DE_B | Open |  |

Output swing setting:

| Header pin # | Pin name for PI3EQX7841 | Switch status | Remark |
| --- | --- | --- | --- |
| JP12 | OS_A | Open |  |
| JP13 | OS_B | Open |  |

- Quick Start — For I2C control:

To start-up the PI3EQX7841 EVB, complete the following steps:

- Short JP9 for power on EVB

- Check the head pin status setting follow Table 1

- Connect the JP6 on EVB to PC’s USB3.0 port through a USB3.0 Type A to B cable.

- Plug the USB3.0 device into EVB USB Type-A connector JP7 through USB3.0 cable

Table 2 Header pin is set as defaulted on EVB

| Header pin # | Pin name for PI3EQX7841 (I2C mode) | Switch status | Remark |
| --- | --- | --- | --- |
| JP4 | EN# | Open | Chip Enable open Chip Disable short to Vdd |
| JP5 | I2C_EN | Short to VDD | Pin strap control modeshort to GND I2C control modeshort to Vdd |
| JP8 | +5V | Short | Device Vbus 5V output |
| JP10 | EQ_A | Open | Equalizer setting on Channel A Programming by I2C |
| JP11 | SDA | Open | Connect to I2C SDA |
| JP12 | SCL | Open | Connect to I2C SCL |
| JP13 | A1 | Short to GND | For I2C addressing A1=0 |
| JP14 | EQ_B | Open | Equalizer setting on Channel B Programming by I2C |
| JP15 | A0 | Short to GND | For I2C addressing A0=0 |

- I2C control setting for Equalizer, De-emphasis and output Swing:

| I2C control for Equalizer |  |
| --- | --- |
| I2C control for De-emphasis |  |
| I2C control for Output Swing |  |

PI3EQX7841 EVB Schematic

PCB Layout

Figure 2, Top view of PI3EQX7841 EVB Layout

Figure 3, Bottom view of PI3EQX7841 EVB Layout