Layout rules for 90ohm differential impedance

Stack-up

| Plane | Material | Thickness (mil) |
| --- | --- | --- |
| Solder mask | Mask paint | 1.2 |
| Signal | Copper | 1.9 |
| Prepreg | 2116 | 4.4 |
| Vcc | Copper | 1.4 |
| Core |  | 47 |
| Vss | Copper | 1.4 |
| Prepreg | 2116 | 4.4 |
| Signal | Copper | 1.9 |
| Solder mask | Mask paint | 1.2 |
| Total |  | 62.4 |

- Use 6-7-6 mils for trace-space-trace for the micro-strip lines (the traces on top and bottom layers) for 90ohm differential impedance.

- Use 6-5-6 mils for trace-space-trace for the strip-lines (the traces inside layers) for 90ohm differential impedance.

- Use FR4.

- Using standard 4 to 8 layers stack-up with 0.062 inch thick PCB.

- For micro-strip lines, using ½ OZ Cu plated is ok.

- For strip-lines in 6 plus players, using 1 OZ Cu is better.

- The trace length miss-matching shall be less than 5 mils for the “+” and “–“ traces in the same pairs

- More pair-to-pair spacing for minimal crosstalk

- Target differential Zo of 90ohm ±15%

The layout guidance for the trace routings

Figure 3, the layout guidance for the trace routings

The PCB layout recommended:

- Use 0.1uf in size of 0402 for all the Vdd (any power pins) pins of the IC device, as close to the Vdd pins as possible, within 2-3mm if feasible.

- Use dedicated Vdd and GND planes for to minimize the jitters coupled between channel trough power sources.