PI5A3157 IBIS Model Check

Two IBIS Models: PI5A3157_C_RevA.ibs and PI5A3157_ZA_RevA.ibs.

1. Model Type

[Pin]  signal_name  model_name

1     B1            TERM1

2     GND           POWER

3     B0            TERM1

4     A             TERM2

5     VCC           POWER

6     S             INPUT

[Series Pin Mapping]

4     1       SWITCH1     1

4     3       SWITCH1     1

Conclusion

1. The PIN and PIN mapping information described in IBIS model is exactly correct according to the datasheet.

2. Switch ON Resistance based on IBIS Model:

Simulation by HSPICE tool with following two conditions: Vcc=3.3v(Defined in IBIS model)

1) Io=24mA, Vin=0V: We get Vout=0.14885V, so Ron=6.2021Ω

2) Io=-24mA, Vin=3V: We get Vout=2.6847V, so Ron=13.1375Ω

Conclusion:

The values of Switch ON Resistance based on Hspice simulation with IBIS model are correct according to the datasheet.

3. Analog Switch simulation result with following circuit diagram:

Input signal frequency is 250MHz, with 10pF pull-down capacitance and 100Ω pull-down resistor to the Output.