PI6CV304 IBIS Model Check

One IBIS Models: PI6CV304.ibs.

1. Model Type

Modify the original PI6CV304.ibs to following two ibis models which are different in packaging, while their pins’ information is the same.

PI6CV304_L_RevA.ibs– 8-pin 173-mil wide TSSOP (L)

PI6CV304_W_RevA.ibs– 8-pin 150-mil wide SOIC (W)

[Pin]  signal_name  model_name 			Pin Configuration

1     CLK_IN			IN

2     OE            IN

3     Y0            OUTPUT

4     GND			GND

5     Y1            OUTPUT

6     VDD			POWER

7     Y2            OUTPUT

8     Y3            OUTPUT

Block Diagram							Function Table

Conclusion

The PIN information described in IBIS model is exactly correct according to the datasheet.

2. Internal Series Resistor based on IBIS Model:

PI6CV304_L_RevA.ibs and PI6CV304_W_RevA.ibs share the same Pulldwon and Pullup data.

1). PullDown: 	TYP:

V = 0.20054V,	I = 0.01232A,		so Rs = 16.27760Ω

V = -0.20054V,	I = -0.01228A,	so Rs = 16.33062Ω

MIN:

V = 0.20054V,	I = 0.011088A,	so Rs = 18.08622Ω

V = -0.20054V,	I = -0.011052A,	so Rs = 18.14513Ω

MAX:

V = 0.20054V,	I = 0.013552A,	so Rs = 14.79782Ω

V = -0.20054V,	I = -0.013508A,	so Rs = 14.84602Ω

2). PullUp: 		TYP:

V = 0.09958V,	I = -0.00614A,	so Rs = 16.21824Ω

V = -0.09961V,	I = 0.00607A,		so Rs = 16.41021Ω

MIN:

V = 0.09958V,	I = -0.005526A,	so Rs = 18.02027Ω

V = -0.09961V,	I = 0.005463A,	so Rs = 18.23357Ω

MAX:

V = 0.09958V,	I = -0.006754A,	so Rs = 14.74386Ω

V = -0.09961V,	I = 0.006677A,	so Rs = 14.91838Ω

Conclusion:

The values of Internal Series Resistor based on IBIS model are quite correct according to the datasheet.

3. Analog Switch simulation result with following circuit diagram:

Input signal frequency is 125MHz: vin	clk_in 0 pulse(0 clamp 0n 0.1n 0.1n 3.9n 8n)

1) Without trace, and add 50Ω pull-down resistor and 5pF pull-down capacitance to the output.

PI6CV304

OUT

SCL_C

SDA_C

50Ω

SCL_C

SDA_C

IN

SCL_C

SDA_C

C

SCL_C

SDA_C

IN

SCL_C

SDA_C

VDD/2

SCL_C

SDA_C

Without Package:

With Package:

2) With 2inch trace before the output

PI6CV304

OUT1

SCL_C

SDA_C

25Ω / 33Ω

SCL_C

SDA_C

50Ω

SCL_C

SDA_C

IN

SCL_C

SDA_C

C

SCL_C

SDA_C

OUT

SCL_C

SDA_C

IN

SCL_C

SDA_C

OUT0

SCL_C

SDA_C

Trace=2inch

SCL_C

SDA_C

a. Add 25Ω resistor, 2inch trace and 5pF pull-down capacitance to the output.

Without Package:

With Package:

b. Add 33Ω resistor, 2inch trace and 5pF pull-down capacitance to the output.

Without Package:

With Package:

4. IBIS Data Graph of PI6CV304_L_RevA.ibs and PI6CV304_W_RevA.ibs

(1)  Model IN: C_comp=4.000pF

a. POWER Clamp

b. GND Clamp

(2)  Model TRI_OUT: C_comp=6.000pF

a. Pullup

b. Pulldown